# Project description:

## Background

### State of the art:

One of the hottest topics in condensed matter physics is the realization of a quantum computer. The main advantage of such a quantum computer would be it’s ability to solve specific classes of algorithms orders of magnitudes faster than classical computers.

As we all know a classical computer is based on deterministic two states called bits. A quantum computer is also based on two level states (basis states) units call quantum bits (qubits). However, a qubit unlike a classical bit exploits the quantum effect of superposition. As a consequence, a quantum system can be simultaneously in both basis states..

There have been several proposals for implementing such qubits (below just solid state implementations are listed):

- Electrons on Helium (He) [1]

- Semiconductors:

    - Nuclear spin qubits [2]

    - Electron (hole) spin qubits [3]

- Superconductors:

    - Flux qubits [4]

    - Charge qubits [5]

As stated above, one of the suggestions, which came in 1998 by Loss and DiVincenzo, was to use the spin of electrons (holes) for the realization of qubits.  The spin is an intrinsic quantum mechanical property of every elementary particle. In a magnetic field the spin degree of freedom lifts the degeneracy of an orbital energy level. The level splits into two, typically labelled as spin-up and spin- down. This two level system can act as a qubit, the so-called spin qubit.

However, for accessing and manipulating the spin degree of freedom, one must first confine the charge into a region, which is in size comparable to the charge particle wavelength. Such a confinement can take place in a so-called a quantum dot (QD). QDs are very small structures (their diameters can reach tens of nanometers) and because of their almost zero dimensionality, the energy levels for a charge particle are discrete and far away from each other. By applying external magnetic field spin energy states splits in two and become distinguishable for manipulation and readout.

A few years after the Loss- DiVincenzo proposal for the realization of a scalable quantum computer, DiVincenzo published a list of conditions  which a qubit should fullfill for a quantum computer to work correctly[6]:

The 5 criteria for quantum computation are:

- identification of well-defined qubits: A well defined qubit is a two level (two state) system whose levels are distinguishable and highly controllable. The qubit operation takes place by operating (manipulating) this two states.

- reliable state preparation: Write sth about it

- low decoherence times (high coherence): Decoherence is phenomena of losing initially set qubit state (can you try to write it in simpler terms?) and is characterized by characteristic time, in literature called coherence time T2 [2].

- For all qubits it is desired to have coherence time as high as possible while gate operation (spin state manipulation) and measurement (state readout) as low as possible

- accurate quantum gate operations (state manipulation) (write sth about it) and

- strong quantum measurements (state readout) (write sth about it)

Decoherence is phenomena of losing initially set qubit state and is characterized by characteristic time, in literature called coherence time T2 [2].

For all qubits it is desired to have coherence time as high as possible while gate operation (spin state manipulation) and measurement (state readout) as low as possible.

QDs are very small structures (diameters can reach tens of nanometers) and because of their almost zero dimensionality, possible energy levels for a charge particle are discrete and far away from each other. This fact has enabled the possibility of confining single charge particle on such a structure. By applying external magnetic field spin energy states splits in two and become distinguishable for manipulation and readout.

Double quantum dot (DQD) (Use this part when you will describe your experiment in a DQD)

For achieving good state preparation, fast manipulation and fast measurement, additional mechanisms are required beyond ones offered by single QDs. One of the most promising building block for the realization of the spin qubit quantum computer based on quantum dots is serial double quantum dot (DQD) system.  A DQD system consists of two neighboring quantum dots tunnel coupled to each other, which simply means that they can exchange charge particles by tunneling. The main physical property which makes them favorable for the realization of a qubit is the Pauli exclusion principle. It says that two identical fermions (in this specific case electrons or holes) cannot occupy same energy state. Two further principles are then based on this one: exchange interaction (exchange interaction is based on coulomb repulsion, do not see its direct relation to Pauli) for spin manipulation and spin blockade for state readout.

Figure 1: Spin state readout based on spin blockade in gate defined electron DQD. The blue circles represent the individual quantum dots, the white lines the gates and the black arrows in the QDs the electron spin in the left and the right dot. Right to the DQD is a charge sensor in form of a quantum point contact [from where is the picture, citation]. In the case of two electrons on the right dot current through the charge sensor does not flow, otherwise it flows (NO!). I think you should not use here the charge sensor. Spin blockade can be observed also in a current measurement. I am afraid that the concepts are getting too much mixed.

Figure 1 describes how spin blockade can be used to to extract information about the electron spin in the left QD in the DQD system. If the spin configuration is like in Figure 1a) then after electrostatic pushing, by applying voltage pulses on gates L and R, both electrons finish on the right dot and current through charge sensor stops to flow, like shown in Figure 1b). In the other case, Figure 1c), electrons on both dots have same spin and due to Pauli exclusion principle they stay in that configuration after electrostatic pushing. Consequently, current through charge sensor continues to flow.  (You need to speak about singlet triplet else it is not clear)

Materials

Until 2005, the mostly used material for the realization of spin qubits was GaAs. However, it has the drawback that electron spins in this material loose their coherence fast because of their coupling to the high number (~106) of gallium and arsenic nuclear spins trough the so called hyperfine interaction. This interaction manifests itself as noise added to externally applied magnetic field (nuclear noise) .The initially measured coherence time by J. R. Petta et al. T2 was 10 nanoseconds [9] by J. R. Petta et al. . By applying a Hahn echo pulse sequence the coherence time was boosted to 1.2 microsecond [9]. By further studying nuclear noise properties and applying techniques for nuclear notch filtering, F. Malinowski and F. Martins from the group of Charles M. Marcus in Copenhagen have recently achieved coherence times of 0.87 milliseconds in gate defined GaAs double quantum dot [7].

Silicon (Si) on the other hand is very promising because it can be isotopically purified and left just with the 28Si isotope which is a nuclear spin zero element. Thusthe nuclear noise can be eliminated and the coherence time boosted. The additional big advantage is its compatibility with current CMOS technology. This could become very important when moving towards the realization of a large number of qubits as required by quantum algorithms.

(speak about the three types of Si quantum dots dzurak, Eriksson,, heterostructures, Morello and what Simmons does with STM. In this discussion it needs also to come out that geometries are getting more and more complex there are people fabrication quintuple dots) One way of defining QDs in silicon is by means of phosphorous P doping in specific Si crystal regions. In that case a phosphorous atom behaves as an electron quantum dot because of its confining potential. Andrea Morello’s Group from UNSW in Australia, by applying a Hahn echo pulse sequence, has measured electron spin coherence times exceeding 200 microseconds, in a non – isotopically purified Si:P system [3]. By using isotopically purified 28Si:P and the nuclear spin of phosphorous atom as a qubit, the same group has achieved nuclear spin coherence time of 60 milliseconds [2].

The major drawback in silicon is the relatively weak spin orbit coupling for electrons which results in difficult spin manipulation via electric fields (as described in more detail in Research Methods). A solution around this problem is to use holes instead of electrons. (speak about the CMOS qubit of the Grenoble team).

Another other way for defining QDs on a silicon based platform, used in our group, is by combining it with germanium Ge [10].The epitaxial growth of Ge on Si can lead to the formation of QDs and and hut wires (a special type of nanowires) due to the different lattice constant between them. Recenty magnetotransport measurement (describe what Hannes has reported in his paper and why is this interesting for the proposal) : Nanowires can be further electrostatically splitted in several QDs, forming double quantum dots or similar structures (Do not think that from the logic this sentence fits here).

Holes in germanium Ge unlike electrons in Si have higher spin orbit coupling which together with keeping above mentioned advantages for electrons in silicon makes them promising spin qubit platform.

Figure 2: SiGe nanowire based DQD sample fabricated in our group by Hannes Watzinger. Ohmic contacts labeled S (source) and D (drain) contacting thin withe nanowire (NW). Gate electrodes (G1 to G5) are on the top, isolated from the nanowire with 10nm hafnium oxide. Show a SEM picture just of the hut wires. Show the preliminary DQD data. Ask Hannes for permission.

In all spin qubit approaches above (and all qubits in overall) there is battle between the spin manipulation time on one side and the coherence time on the other side. For making a set of quantum operation correctly, the manipulation time for one operation need to be much shorter than the coherence time.  (Let’s discuss the three ratios you give below)

π/2 spin rotation achieved in Andrea Morello Group from UNSW Australia in Si:P material system, using ESR (electron spin resonance technique) is 75 ns [3]. Their T2 is 200 us [3]. Do you compare the same T2?

Using hole spin in p-type silicon industrial CMOS as qubit basis, R. Maurand from S. De Franceschi group in CEA Grenoble, achieved π/2 spin rotation of approximately 3 ns [11]. While coherence time T2 in their case was 245 ns [11].

As mentioned above, reason for slow spin manipulation (long π/2 spin rotation) in case of silicon material system is low spin orbit coupling for holes and even lower for electrons, what can be seen from the measured results above. Holes in germanium have much higher spin orbit coupling which should lead to much faster spin manipulation in these type of qubits.

Different type of measurement techniques are applied in order to measure the state of a spin qubit and extract the coherence times:

- DC current readout

- Differential measurement (AC current readout)

- Ohmic reflectometry

- Gate reflectometry

The DC current readout is sensing the electron transport through the qubit by means of current measurement.It is prone to low frequency 1/f noise and BW is low because of heavy filtering for achieving low effective electron temperatures.

Differential measurement (AC current readout) has the same drawbacks as DC current readout. It is typically done with low frequency lock in technique. (Maybe you can comment on the speed of the lockin measurement)

Ohmic reflectometry is a technique of indirect sensing the change of the qubit impedance by monitoring the amplitude or phase of the reflected from the qubit wave (See figure 3 for a more detailed explanation). It is usually done by high frequency lock-in techniques and is not prone to 1/f noise.

Finally, gate reflectometry is a technique of indirect sensing of the change in the qubit capacitance by monitoring the amplitude or phase of the portion of the sent wave reflected from the one of the qubit gates. It’s big advantage is that it does neither require charge transport through the qubit nor the existence of a no charge sensor.

What is reflectometry?

Figure 3: Basic principle of ohmic reflectometry. CS and RS constitutes the equivalent electrical schematic of a measured sample, e.g. a single electron (hole) transistor as charge sensor (I would not say that this is a charge sensor, here you give a general description what reflectometry is) of DQD (also no DQD, this is a general description), formed by single quantum dot. S and D denotes source and drain contacts of the single hole transistor, respectively. The resonance circuit formed with an inductor L and a capacitance C is connected to the source contact.

Reflectometry is a readout technique based on the change of the wave reflection coefficient Γ. It comes from the electromagnetic wave principle – if a wave is travelling in a media with impedance Z0  (e.g. coax cable) and it encounters a change of impedance (e.g. coax end) to Z, a portion of the wave will be reflected back according to the expression: Ar=Ain*Γ,    Γ=Z-Z0Z+Z0  , where Ar is amplitude of a reflected, Ain amplitude of an incoming wave and Γ is reflection coefficient.

By putting a resonant circuit connected to a single hole transitor  (Figure 3, red polygon) instead of coax cable end one can measure charge sensor impedance change caused by change in charge configuration in the nearby DQD (see above about double quantum dot; give a more general description. If elements of a resonant circuit – inductance L and capacitance C are properly chosen, on the resonant frequency of that circuit, f0=1L*C , wave reflection coefficient Γ  is minimized. Proper choosing means that the impedance of complete matching circuit on resonance is equal, or matches, characteristic impedance coax line Z0.   Thus if DQD charge configuration changes -> charge sensor impedance changes -> reflection coefficient changes -> amplitude of reflection wave changes. (It is nicely explained, however as stated above write it in a more general way)

Measurements need to be sensitive enough to achieve a high enough signal to noise ratio (SNR) (typically SNR of more than 10) in a short time. Signal to noise ratio is the ratio of signal and noise amplitude in a given bandwidth. Measurement sensitivity is a measure of the change in an amplitude of current or amplitude (phase) of reflected wave when charge configuration is changed. Noise comes from 1/f noise on lower frequencies, intrinsic shot noise, thermal noise, noise in in measurement equipment… Lowering the measurement bandwidth (integration or filtering) noise is lowered and SNR raised but measurement become slower. Thus for achieving good SNR in short time signal need to be high. In our case fast measurement is required to obtain good quality measurement fast enough. (I think this part does not fit here, should be put when you explain how you will tune your reflectometry what is important)

### Definition of the problem:

As mentioned before DQD are defined by gates positioned on top of the nanowire (Figure 2). Static voltage on gates G1 and G2 create the potential wells forming the double quantum dot system. For operating this DQD as a qubit it needs to be isolated from the ohmic contacts, which can be achieved by G3 and G5.

Since the charge transport through the DQD is forbidden, all readout techniques based on charge transport are not applicable since no current is flowing through the device. (I doubt that you would be able to see sth even if you integrate long enough. How long would you need to integrate to see a let’s say 0.1fA current signal?). Readout techniques in this category are DC current readout, AC current readout and ohmic reflectometry.

A usual solution to this problem is to place next to the DQD an additional, separated quantum dot in the form of a single electron (hole) transistor or quantum point contact, called charge sensor (Figure 1). The charge sensor is electrostatically coupled and thus sensitive to the charge configuration in the DQD. The charge sensor itself is well coupled to ohmic contacts thus it is suitable for charge transport measurements and ohmic reflectometry.

However, charge sensors suffer from conductance profile thermal broadening what lowers the sensitivity thus speed of readout. They also need additional compensation gates to substract the influence of the qubit gates on their conductance. Also, by looking into the future, for the realizatioin of a usable quantum processor, the qubit number needs to be drastically scaled up to achieve a large enough number as required by quantum algorithms. Gate reflectometry does not suffer from previously listed problems and since it is using already defined electrostatic gates it does not need a charge sensor, thus has a big potential to address the scalability problem. Furthermore, by using frequency division multiplexing, all qubits in a quantum processor could be read out through a one (or several) wires. (Should you discuss here frequency crowding? What does the difference in resonant frequencies need to be?)

I think you should start with the work of Reilly and some other groups and then say what is the work with the highest achieved sensitivity sofar. Last year, M.F. Gonzalez – Zalba et al. reported a charge sensitivity of 37 μeHz-1/2 by using gate reflectometry for silicon nanowire based DQD device [12]. The reported sensitivity is similar to that achieved with ohmic reflectometry in charge sensors (RF quantum point contact and RF single electron transistor) which is in μeHz-1/2 regime [12], but suffers from all the issues stated above.

### Proposal objectives:

The objectives of this proposal are to design a fast gate reflectometry system which will be used in order to study the LD qubit created in a a germanium based, double quantum dot.

For the gate reflectometry, the goal is to achieve a charge sensitivity comparable or even faster than that reported in [12]. Such would allow us to have a high bandwidth system necessary for the qubit read out (do you wanted to say sth different?)

After the gate reflectometry set up will have been set up the focus will go to the realization of the LD hole qubit in a DQD structure. The first measurements to be performed are the ones for determining the spin relaxation time T1. the time during which the spin stays in the excited state before relaxing to the ground state.  Subsequently experiments in order to investigate the coherence time of the qubit are going to be performed. More concretely, spin manipulation experiments for measuring the spin dephasing time T2*, the spin coherence time using Hahn echo technique T2, and the spin coherence time using the CPMG pulse sequence technique T2 CPMG, are going to be conducted.

### Working schedule:

#### Designing initial version of reflectometry setup: sample holder, readout circuit, instrumentation setup

Sample holder

One of the key measurement requirements is to lower the electron thermal energy to be able to resolve energy level splitting needed for confining one charge particle spin (what do you mean by this?). For that reason it needs to be cooled down to low temperatures. In this case it is 4 Kelvin (K) or below.  For the initial version of measurement system 4 K dewar with liquid helium has been used.

Why don’t you start with sth like. In order to tune the gate reflectometry system measurements will be initially performed at 4k. Such temperatures are needed for…. During the first year of my PhD I have already prepared a 4K dip stick for such reflectometry measurements (then you can describe the stick). I would not now go into double quantum dots. When you will characterized the gate reflectometry initially you will start with single quantum dots and then you move towards double quantum dots.

Double quantum dots samples are grown on silicon wafers and then cutted in 5x5 mm pieces. They need to be dipped into the liquid helium dewar for cooling. For this purpose Plexiglas sticks (Figure 5), were used. The sample can be positioned on the top of the stick on the so called sample holder. Since, electrical signals needs to be delivered and afterwards measured from the sample, the sample holder is done as printed circuit board (PCB) which routes all the electrical signals to and from the sample. From the room temperature instruments, DC electrical signals are sent through the low thermal conductive twisted pair wires finishing in a PCB connector and radio frequency signals are sent through coaxial cables. Going from PCB DC connector, DC signals are low pass filtered with sourface mounted RC filters (Figure 4) to reduce thermal noise from the wires. After low pass filtering, DC signals are routed to the gold plated bonding pads around area in the middle of the PCB (sample area) on which a typically 5x5 mm sample is glued with silver paste (Figure 4). Electrical contacts from PCB bonding pads to on the sample bonding pads electrically connected to the DQD gates are achieved by wedge wire bonding technique. RF coaxial lines are finishing on the PCB mounted SMP connectors. After SMP connector, using bias tee, DC signal is added to the RF signal.  From there signal is routed to the PCB bonding pads. SMP connectors and bias tees can be seen on Figure 4. Further these signals are connected to DQD gates with the same wedge wire bonding technique.

Figure 4: Initial version of the PCB sample holder. The top figure show the upper view of the PCB board while the lower figure focuses on the back side.

Readout circuit

To measure the charge state of the nanowire single hole transistor (SHT), ohmic reflectometry technique was applied. For that purpose the RF signal was sent down the coax line (Figure 5, right). The reflected signal was separated in the directional coupler and directed to the Weinreb’s CITLF2 and Minicircuits ZX60-33LN-S+  amplifiers (Figure 5, right) to make the SNR immune to the next room temperature stages (explain a bit more). The amplitude of the reflected signal from the resonator circuit depends on the single electron transistor charge state. The explanation of the working principle of ohmic reflectometry can be found at the end of the “State of the art” chapter. The used resonator circuit consisted of a matching circuit (Figure 4) and the SHT resistance RS in parallel to the parasitic capacitance CS, as can be seen in a simple circuit model in Figure 3. Finally there exists always a parasitic capacitance, which is a capacitance to the ground that comes from bonding wires, the sample itself, the RF line and the used inductor.

(There is nice circuit schematic in the Natalia paper. Since she is not explicitly telling that lowering parasitic capacitance increases sensitivity, I was thinking, maybe it would be good to take that circuit model with a reference and make a simulation in Qucs showing that with lowering parasitics, change in reflection coef increases. For the purpose of justifying lowering parasitic capacitance by removing grounds in the PCB- This seems like a good idea!)

For the purpose of minimization of this parasitic capacitance and thus increase sensitivity of the reflectometry, the PCB ground plane is removed below the RF lines, the corresponding PCB bonding pads and the sample. 
Matching circuit elements used are surface mounted inductor Murata 1,2 μH and varactor MACOM MA46H070-1056. A Varactor – a voltage tunable capacitor - was used to be able to always achieve good matching condition despite the change of the SHT resistance Rs following the approach in [13]. (Put this in reflectometry explanation and here just refer to it)Matching condition is situation in which large resistance (~100 KΩ) of the single hole transistor (SHT) is transformed to near 50 Ω value what is characteristic impedance of the RF line, thus minimizing reflected signal amplitude (as can be seen in the reflectometry explanation at the end of the “State of the art” chapter) (do we need to say it again?). For this case change in a reflected signal amplitude due to SHT charge configuration change is maximized and consequently measurement sensitivity is maximized [13].

Figure 5: Plexiglas dip-stick used for cooling down samples mounted on the PCB to 4K. The left picture shows the whole, , while right is the zoom in, highlighting the directional coupler (up) and low noise Minicircuits ZX60-33LN-S+ RF amplifier (down). An additional low noise cryogenic RF amplifier CITLF2 from Sander Weinreb’s Caltech Microwave Research Group can be added in order to increase the SNR of the measured signal.

I would add here the picture of your ohmic reflectometry measurement.

I should read up to here, right?

#### Germanium nanowire based, hole spin single quantum dot tuning and characterization with initial version reflectometry setup

  - Put comparison reflectometry vs current measurement on Hannes sample (diamond graphs)

#### Second generation of the reflectometry setup – dil fridge, UHFLI, AWG, QTLab, new PCB (pict of Iron Man), additional DC filtering, low thermal conducting NbTi coax with attenuators (pict of the probe).

  - TO DO:

#### Moving to the gate reflectometry – Adapting existing setup (readout circuit) for gate reflectometry, main points about gate reflectometry (from Hypoteses, look [12]), our sample considerations: gate coupling to the QD, comparison with [12]

#### Optimizing (boosting) reflection parameter Γ regarding to:  sample holder RF design, readout circuit configuration

  - Sample holder RF design – achieving small parasitic capacitance by engineering sample holder (PCB), achieving good 50 Ω on PCB RF lines and distributing them for achieving frequency division multiplexing

  - Readout circuit optimization – choosing appropriate inductor values L (Figure 3) while keeping parasitic capacitance small like stated above, optimizing directional coupler, amplifiers and filters configuration

#### Implementing setup for spin manipulation measurement

#### Measuring relaxation time

#### Spin dephasing time experiment

#### Spin coherence time experiment:

      - Hahn echo

      - CPMG pulse sequence

### Innovative aspects:

In our group we are working with a germanium nanowire based hole spin double quantum dot. While other groups work with structures based on electron spin in gallium arsenide, electron spin in silicon and Si:P and hole spin in silicon, this particular approach is not yet investigated. It has promising theoretical proposals (put in which proposals and some numbers maybe) in terms of qubit spin state manipulation and coherence time. Easy and fast spin state manipulation is expected because of in situ present large spin orbit coupling for heavy holes in Ge which enable fast spin manipulation by applying oscillatory electric field to particular qubit gates (Figure 2) eliminating necessity for oscillatory magnetic field. This means reducing fabrication complexity because nothing else is required except of already defined gates. Together with all the fabrication reduction complexity brought by gate reflectometry (as explained in Definition of the problem) this approach has high chances of addressing scalability issue.

Relatively long coherence time is expected because of, before mentioned, low qubit charge spin coupling to the surrounding Ge and Si nuclei spin (low hyperfine interaction).

Gates in our DQD system (Figure 2) are positioned directly on the top of the nanowire in which quantum dots are formed. This implies high capacitive coupling between gate and quantum dots. This further implies high sensitivity thus speed of gate reflectometry, as explained in Research methods (or work schedule).

### International collaboration:

We are collaborating with spin qubit team in Charles M. Marcus laboratory in Copenhagen, lead by Ferdinand Kuemmeth. Since they are mature group with a big knowledge in instrumentation and spin qubits in overall, this collaboration helps us a lot in setting up our measurement setup. It would be helpful to visit them several times throughout the year. Here I would like to ask for the finances to cover the trip and accommodation costs for that purpose. The other significant collaboration is with J.J. Zhang in Beijing, China. He is material scientist providing us with silicon germanium nanowire samples.

### Overall goal of the project:

To implement one of the Loss and DiVincenzo’s criteria:

- strong quantum measurements,

and conducting experiments of spin manipulation.

Strong quantum measurements can be achieved by implementing gate reflectometry in our type of qubit structures. After having state readout solved, spin manipulation experiments can be done by applying bursts of microwave signal on electrostatic gates  (as explained in research methods).

Spin manipulation experiments will be guideline for achieving second of the Loss and DiVincenzo’s criteria:

- accurate quantum gate operations

## Specific aims

### Clear aims:

Samples are done in cleanroom… (take from someone in group)

All experiments are done on DQD and TQD samples placed on the printed circuit board (PCB) sample holder (put the picture) in the dilution refrigerator with a base temperature of 10 mK.

Fig. x1. PCB holder (green) with mounted nanowire based sample (middle, grey) fabricated in our group by Lada Vukušić. Altogether mounted on golden plated copper fork on the dilution fridge insert. Copper wires are coaxial cables providing high frequency connection for spin manipulation and readout. Nanometer gates and ohmic contacts on the sample are connected by wedge wire bonding.

Electrical connection with the sample is achieved through thermally low conductive looms for DC signals and coaxial cables for RF and microwave signals. (put the picture of the probe) All cables finish in PCB connector and further electrical contact with the sample is achieved by wedge wire bonding.

On room temperature side there are several instruments for sending and receiving the signals from the sample. Firstly, DQD and TQD needs to be tune in correct electrostatic configuration what is achieved through the low-noise, optically isolated, voltage DC sources.

Sequences of high-speed pulses (ranging from hundreds of nanoseconds to several milliseconds)  coordinated together with bursts of microwave signals (several GHz up to several tens of GHz) are sent via coaxial cables to manipulate DQD and TQD charge and spin state thus providing spin qubit manipulations.

Pulses are generated by arbitrary waveform generator (Tektronix AWG5014C) and microwave signals by microwave signal source (Rohde & Schwarz SMF100A).

Qubit state is read-out by probing radio-frequency (RF) signal reflected from resonant circuit consisted of discrete inductor and capacitor and gate capacitance between DQD and TQD top gates and confined charge area in those. Probing is done by high frequency lock-in measurement technique using Zurich Instruments UHFLI lock-in amplifier.

### Hypoteses:

(Don’t know what to put here and what in research metodology)

Main hypotheses is that our gate reflectometry is sensitive enough to achieve fast quantum state readout.  Read out parameter (one which needs to be boosted)  by gate reflectometry is resonance frequency shift ∆f due to hole tunneling form one to another dot in DQD or TQD system: 				   ∆f  ≈ CQf0/2Cp.   Resonant frequency, f0=1L*Cp, of resonance circuit depend on externally added lumped inductance L which is externally added, and parasitic capacitance Cp. Because L is easily tunable and Cp can be reduced to some level by engineering, main hypotheses is that quantum capacitance due to a hole tunneling, CQ is big. It is given by our sample and we expect it to be relatively high because of the following reasons.

(This need to be changed according to Csigma)

CQ depends on capacitive coupling of reflectometry readout gate to QDs in a qubit Cg, and parasitic capacitances Cp, according to:  CQ∼CgCΣ2   ,     CΣ ≈  Cg+ Cp

Previous expression suggest that pathway for boosting sensitivity of gate reflectometry is to have Cg high and Cp low.

Since in our types of structures gates are positioned on the top of the nanowire (d is small, l and w are relatively large) consisting QDs (Fig 1.) we expect high Cg ,according to:  Cg≈ ε0* εroxide*l*wd

Small parasitic capacitance we are going to achieve by engineering our sample holder (PCB). Isolating PCB sample area from the ground by removing ground planes and decoupling RF and DC ground by putting relatively large resistors in DC line around that area. On Fig.x.1. around the sample PCB is translucent indicating that there is no copper ground plane.

Figure 1. Nanowire based single quantum dot, predecessor of double quantum dot on Fig.x.

## Research methods

Here we are proposing integration of two qubit Loss and DiVincenzo’s criteria in our type of qubit. First is qubit state (spin) readout. Other one is spin state manipulation.

### Qubit state readout:

Figure 2. Gate reflectometry schematic on the DQD sample from Fig.x1. LC resonators are connected to three of the gates. Signal from different gates are distinguished by frequency multiplexing since resonant frequencies are different because of different inductor values.

What is reflectometry?

Reflectometry is readout technique based on change of wave reflection coefficient Γ. It comes from electromagnetic wave principle – if the wave is travelling in media with impedance Z0  (e.g. coax cable) and it encounters change of impedance (e.g. coax end) to Z, portion of the wave will be reflected back according to the expression: Ar=Ain*Γ,    Γ=Z-Z0Z+Z0  ,  ᵠAr-ᵠAin=ᵠΓ, where Ar is amplitude and ᵠ( Ar) phase of the reflected, and Ain amplitude and ᵠ(Ain) phase of incoming wave. Reflection coefficient phase is ᵠ(Γ).

Putting resonant circuit with incorporated device instead of coax cable end one can measure change in impedance (capacitance) of that device. If elements of a resonant circuit – inductance L and capacitance C are properly chosen, on the resonant frequency of that circuit, f0=1L*C , wave reflection coefficient Γ  is minimized and it’s phase has inflection point and highest slope, Fig 2. top right blue and red.

In case of resonant LC circuit consisted of just L and C with very small R, Z is almost purely imaginary consisting of inductive and capacitive reactance. Thus if capacitance of sensed device changes -> ᵠ(Γ) changes and so phase of reflected wave  ᵠ( Ar). Thus, by measuring phase of reflected wave ᵠ( Ar) and comparing it with ᵠ(Ain)  one can get information of the impedance (capacitance) of sensed device.

Fig 3. S11 parameter vs frequency for different capacitors (put something from qucs instead of this one)

Our plan:

RF wave (tens to hundreds of MHz) is generated and sent from UHFLI out port down the coax cable.  Going through directional coupler and encountering three resonant circuit frequency multiplexed on different resonance frequencies by choosing different values for surface mount inductors L1, L2, L3. Each of this inductors will be wire bonded to finger like gates, as shown in Fig 2. Here is an example for nanowire, double quantum dot based qubit.  Gates LP (left plunger) and RP (right plunger) are capacitively coupled to the left and right quantum dot respectively. When electron undergo tunneling between the dots there is an onset of quantum capacitance, changing overall capacitance seen by the resonant circuit, which changes resonance frequency (according to expression for f0) and consequently amplitude and phase of reflected wave which is then measured.

### Spin state manipulation:

Write something about it

# References:

J.I.Colless, Dispersive Readout of a Few-Electron Double Quantum Dot with Fast rf Gate-Sensors”

Some lecture: http://www.physics.udel.edu/~msafrono/650/Lecture19.pdf

- Platzman, P. M.; Dykman, M. I.; Science 1999, 284, 1967-1969

- Jarryd J. Pla et al., Nature 2013 496, 334–338

- Jarryd J. Pla et al., Nature 2012 489, 541–545

- A. Morello et al., Nature 2010 467, 687

- Xiaobo Zhu1 el al., Nature 2011 478, 221–224

- H. Paik et al., Phys. Rev. Lett. 2011 107, 240501

- David P. DiVincenzo,  arXiv:quant-ph/0002077v3

- F. K. Malinowski; F. Martins; P. D. Nissen; E. Barnes; Ł. Cywiński; M. S. Rudner; S. Fallahi; G. C. Gardner; M. J. Manfra; C. M. Marcus; F. Kuemmeth,  arXiv:1601.06677

- J. R. Petta et al., Science 2005 309, 2180

- H. Watzinger et al., arXiv:1607.02977

- R. Maurand et al.,  arXiv:1605.07599

- Gonzalez-Zalba, M. F. et al., Nat. Comm.  2015 6, 6084

- N. Ares et al.,  Phys. Rev. Applied 2016 5, 034011

G. Katsaros,  Marie Curie proposal